A TSR Harddisk and COM

Monitoring System.

26HDCOM (C)1991 Cornel Huth

SUMMARY FEATURES ----------------------------------------- 12/06/90-pre

26HDCOM.COM, HC for short, requires an IBM PC, XT, AT, AT-386, or other

close compatible and DOS 2 or above. It's a TSR program that monitors

the hard drives (D: also on ATs only) by reporting the drive, cylinder,

head, sector, and status of the disk controller. HC can also monitor

the serial ports/modem and will report DTR,RTS,CTS,DSR,RI,DCD and also

bps and communication protocol (parity, data & stop bits).

What makes HC even more useful is that it displays the status line on

the 26th line of IBM compatible video systems, including the CGA, EGA

and VGA color systems. For IBM compatible monochrome systems, the status

line is displayed on line 1. This status line can be momentarily disabled

by having Scroll Lock on, or by pressing the ALT key down.

STATUS LINE ---------------------------------------------------------

C:1023 Hd:15 Sc:63  OK                         DTR RTS CTS DSR RI  DCD 38400 N81

is what the status line looks like full-blown.  The hard drive letter

is followed by the current cylinder, head, sector, error message, and

then the serial port status.  On color systems this is black with a green

background.  On monochrome systems it's in reverse video.

Possible disk errors are (including floppy):

0  OK

1  BAD COMMAND

2  ADDRESS MARK NOT FOUND

3  WRITE PROTECT ERROR

4  SECTOR NOT FOUND

5  RESET FAILED

6  DISKETTE REMOVED

7  BAD PARAMETER TABLE

8  DMA OVERRUN

9  DMA ACROSS 64K BOUNDARY

A  BAD SECTOR FLAG DETECTED

B  BAD TRACK FLAG DETECTED

C  BAD MEDIA TYPE

D  TOO MANY SECTORS ON FORMAT

E  CTRL DATA ADDR MARK FOUND

F  DMA ARBTRA`N OUT-OF-RANGE

10  UNCORRECTABLE ECC|CRC ERR

11  ECC CORRECTED DATA FAILURE

20  GENERAL CONTROLLER FAILURE

40  SEEK FAILED

80  TIME OUT|DRIVE NOT READY

AA  DRIVE NOT READY

BB  UNDEFINED ERROR OCCURED

CC  WRITE FAULT

E0  STATUS ERROR/ERROR REG = 0

FF  SENSE OPERATION FAILED

A disk error will be retained on the status line for 2 seconds

provided that no new error occurs within that time.

The COM section monitors:

DTR Data terminal ready

RTS Ready to send

CTS Clear to send        &   bps rate

DSR Data set ready           parity

RI  Ring indicator (yours)   data bits

DCD Data carrier detect      stop bits

USAGE ---------------------------------------------------------------

HC occupies about 2500 bytes of system RAM with a standard DOS

environment when resident and is controlled by command line options.

HC       * install TSR to monitor status

HC /U    * remove from memory

HC /S    * sleep mode

HC /R    * restore status line and awaken if in sleep mode

HC /Cn   * monitor COM port (n=1-4  0=off)

HC /Mn   * select controller BIOS data interpret mode (n=1-4)

HC /H    * help

At the COMMAND prompt, enter HC.  This installs the TSR portion.

There are currently three different interpretation modes supported for

the harddisk BIOS data area.  HC will start up in mode 1 (generally

XT controllers).  If you have an AT use mode 3 (drive 0=C:) or 4

(drive 1=D:). If this doesn't work with your AT, try mode 2 (non-BIOS

controller card).  To change the mode that HC starts up in, use a

disk editor (Norton's, PC Tools, etc.) and change the byte at 113

decimal (71 hex) from '1' to the mode you want (1 to 4).  This is

the character '1' to '4', not ASCII 01 to 04.  If you don't want

to change it permanently you can still change it with the command

line option /M2, /M3, /M4.

BIOS supports 4 COM ports.  To select the port to monitor, use HC/Cn

where n equals 0 - 4.  0 will turn off any current COM monitoring.

HC can be removed from memory (HC/U).  This should only be

done when at DOS's top level, not from a shell, and really only

when it is the last current TSR installed.  I say 'really' because

HC checks to make sure that the interrupt it uses still points to

itself, i.e., no other TSR is using it.  You can safely try HC/U at

anytime (at top level); HC will only remove itself if safe to do so.

A better way to 'disconnect' HC is to use the sleep mode /S.

Sleep mode (HC/S) is performed by unchaining HC from the user timer-tick

interrupt that it uses (1Ch) and replacing the interrupt vector to the

one active when HC was installed.  This means that HC can be 'turned off'

even after shelling to DOS from another program.  Very useful.  And of

course, HC can be restored to its active state with /R.

The restore function (HC/R) awakens the dormant HC and also resets the

display to 26 lines on color systems, since any video mode change

(e.g., MODE co80) will reprogram the system to its default mode.  Note

that EGA or VGA systems must be in standard mode, i.e., 25 lines,

not 43 or 50, to either install or restore.  The VGA should also be

connected to a VGA monitor (720x400 is the std text mode of the VGA).

If you are in a non-compatible mode, HC will not install or restore

itself but just exit with an error message.

It sure is nice to know what your harddisk and modem are up to.

The MASM assembly source is included for your computing pleasure.

chh

12/03/90 2.00 adapted from DSTCOM v3.10 - chh

12/06/90 2.01 Modify bps to handle above 9600, e.g. 38400 - chh (4Geo)

12/06/90 2.01 fix mode1/2 positioning of head number on stat line - chh