For those interested, this is a copy of a reply to a forum member who asked

about programming QIC drives.  Since I've been getting a lot of these

requests lately, I decided to upload this reply so I don't have to keep on

typing it!  (I subscribe to the laziness theory...)

============================================================================

It's a long road, any way you look at it.  I can give you some basics but to

cover everything would require a novel.

The standard floppy controller uses base address 3F0h, DMA 2 and IRQ 6.  A

relatively new book called "The Undocumented PC" by Frank Van Gilluwe has

some excellent examples of DMA programming and FDC programming.  It would be

well worth your money to buy it.  Also you should get the Intel data sheets

on the 82077 and 82078 floppy controllers.  All it takes is one phone call to

a number I can't remember and they send the info right out.  Intel also has 2

forums on CIS -- INTEL and INTELACCESS.  I believe you can request literature

there as well.  You'll also need documents QIC-40 and -80, and 3010 and 3020

for the new drives which aren't in production yet.

You have to initialize the 82077 using the CONFIGURE command to set the FIFO

threshold to 16 bytes.  Then you have to issue the SPECIFY command to set the

step rate time, head unload time and head load time.  These values will

change depending on the data rate you're using so if you change the data

rate, re-issue the SPECIFY command with the proper values.  The specs call

for a step rate time of 2ms, and the value you have to send on the SPECIFY

command will change with the data rate in order to KEEP the step rate time at

2ms.

You'll need a fairly accurate way to measure timing resolutions at the 1ms

mark.  I use the 8253 timer.  Others cry about this but I haven't had any

problems doing it, and it has the virtue of not breaking down on faster CPUs.

The time I'm using is a 4ms delay after every drive command and it seems to

work pretty well.

Once you get the FDC basics down, you have to keep track of which cylinder

the FDC thinks the tape drive is on.  To issue command n to the tape drive,

you have to issue an FDC SEEK command n tracks away from the current track

(from the FDC's point of view).  If the FDC after initialization thinks the

tape drive (which it thinks is a floppy) is on track 0, to issue a command

10, seek track 10.  To issue a command 18 after that, seek 18 tracks forward

from 10 to track 28.  After you hit around 120 or so it's a good idea to

reverse directions and start seeking tracks downward instead of upward.  And

as you get close to 0, reverse again and just keep this cycling process

going.

To issue a report command, you first issue the main report command to the

drive.  Then, depending on whether the specific report command is 8 or 16

bits, you have to loop 10 or 18 times, throwing out the results of the first

and last iteration: issue drive command 2, REPORT NEXT BIT, then issue the

FDC command SENSE DRIVE STATUS.  This will return the FDC's status register 3

value; bit 4 in this value contains the track/0 status which will be used as

the next bit in the report command.  So you have to do this 10 or 18 times

and keep accumulating bit 5 out of the SR3 value.  It's the longest process

in the entire program, to handle report commands.

When you're accessing the tape, remember that all sectors on the tape have to

be addressed like a floppy.  All this addressing information is in the

QIC-40, -80, -3010 and -3020 docs.  But you'll need routines to convert tape

relative addressing to floppy relative and vice versa because all the

pointers that are on the tape itself are tape relative, but to program reads

and writes through the FDC you'll always have to work in floppy relative

parameters.

Now, the hard part.  ECC.  If you can find somebody who knows it, you're

miles ahead of where I was.  When I got to the point of doing ECC, I called

Stac Electronics -- the only source for QIC standard ECC routines -- and they

wanted $30,000 (yes, thirty grand) for 4 little routines that did the job.  I

laughed all the way to the telephone and began a 5 month hell that I would

never want to repeat again in my life.  Calling here, there and everywhere,

looking through books, cold calling electronics design firms, ANYTHING I

could do to get some kind of info on how to do the ECC.  I finally drove up

to Boulder and picked up a book that I didn't understand; out of desperation

I tried calling the author at the U of Hawaii. By an act of God, it turned

out he was giving one of 2 seminars that year right here in Denver, 3 weeks

away.  It was $1300 out of my pocket but I signed up and went.  It took about

3 months after that seminar, and many long tired hours of trying to fax

questions to Hawaii, until everything finally worked.  By now, I don't touch

those routines with a ten foot pole.  They work, they're to standard, and I

leave them the hell alone.  To do anything QIC compliant, you have to have

the QIC standard ECC or something better if compatibility is not an issue.

Fastback uses a different code that allows up to 50% of the data to be

recovered but it means that 50% of your tape space is used up for ECC parity

bytes.

Anyway, that's a start.  I'm not making it easy for you; you'll have to do

your homework and learn the hard way like I did but having somebody to guide

you, you're doing better than I did.  There just aren't any real resources

around for doing this kind of thing.  It's very, very difficult.  I've only

covered the tip of the iceberg; every drive has abnormalities and quirks and

special requirements that you'll have to find out as you go along.

If I worked straight through without interruption, it would have been about

10 months from the start date just to get the core drivers into alpha test.

And that's working at home, which is usually 7 days a week, averaging 12 to

14 hours a day.  Just know what you're getting into.

I will be licensing my drivers very soon if you want an alternative.  Each

case is handled specifically.  I'll need to know the details of your product

and your distribution and we can talk from there, via email of course.  I'll

be hanging around!

-- Chris

Oh yeah, one thing nobody has documented: all drives except Iomega require a wakeup sequence of command 23 then 20 (which are listed as reserved in QIC-117) to activate the drive; 46 then 02 for CMS drives.  I forgot what the deactivate numbers are.  I wrote that so long ago I haven't looked at that code in over a year.  I'll let you know.  But the drive won't do a thing until you issue those commands.

Good luck!!