-----------------------------------------------------------

--                                                       --

--                 Save PCX Version 1.00                 --

--                                                       --

--  (C) Copyright 1992, Prometheus Systems Incorporated  --

--                                                       --

--                Save PCX User's manual                 --

--                                                       --

-----------------------------------------------------------

Introduction.

-------------

SavePCX is a very small (less than 3Kb resident) which allows you

to capture images from the graphics screen, and save them as PCX

type pictures.

Status.

-------

SavePCX is Freeware.  It is completely free... however, SavePCX is

still copyrighted and you are not allowed to make any changes to the

program or the documentation.  You are allowed to distribute this

program, but you are not allowed to sell it, you have to GIVE the copy

away, you may not regain any losses made by distributing this program.

Also, SavePCX may NOT be combined in a package of any sorts unless the

package is COMPLETELY free.  If you give a copy to a friend make sure

he gets this documentation too.

Installation.

-------------

Since SavePCX is very small in size, this utility should not cause

any problems, even in systems with little memory.

Just type SavePCX at the DOS prompt to load SavePCX in memory.  No

parameters are needed.

To load SavePCX in the UMB, type

LH SavePCX               or

LOADHIGH SavePCX

Be sure you have an UMB manager loaded (EMM386.EXE, QEMM.SYS,

QRAM.SYS...).

If you try to install a second copy of SavePCX, a message (Already

in memory) will appear on screen and the second copy will not be

installed.

How it works

------------

SavePCX is a resident screen-capture utility. Once loaded, it

will remain in memory and can be activated by pressing the "Hot

Key" (Ctrl+Alt+S).

Once activated, SavePCX will check the video mode and save the

graphics screen into the current directory. The file is placed in

the current drive and path.  If there is no more room for the

file the program will do it's best to close the file and return

you to your task.

Every file made by this utility can be recognized by the name

PSInnnn.PCX where 'nnnn' represents a number from 0000 to 9999.

An existing file cannot be overwritten by this utility, it first

checks the filename, if the name already exists it increments the

number and checks again until the filename doesn't exist.

Operations.

-----------

The operation of SavePCX is easy and can be done in a few simple

steps.

First load SavePCX in memory.

Once loaded, a message that SavePCX is now resident or was already

resident will appear on screen and you will be returned to the

DOS prompt.  Except for a small reduction in system memory (3Kb),

you may continue using your computer normally.

Second, now that SavePCX is an extension of DOS, it may be

activated while in any program. The way for doing this is as

follows:

Hot Key: This combination of keys is used to activate the program.

To avoid interfering with the current program, the keyboard will

respond normally until these keys are pressed. Once the hot key

is pressed, the current operation is suspended and the screen is

saved on disk (current drive and directory).  The hot key for

SavePCX is Ctrl+Alt+S.

What is saved.

--------------

SavePCX will do it's best to identify the active video mode and

save what you see on screen.

Following video modes are supported.

Mode 04h : CGA 320*200, 4 colour.

Although this CGA mode can make use of any of four

different colour schemes, due to the limitations of the

PCX format version 3.0, the correct colour scheme will

not be saved.  This will usually not be a problem since

most drawing packages only use the Cyan/Magenta/White

(CMW) colour scheme.

Mode 05h : CGA 320*200, 4 gray scale.

This video mode is technically the same as mode 04h.

Again due to limitations in the PCX format version 3.0,

the gray scales are not saved.  The picture is saved

just like it was mode 04h

Mode 06h : CGA 640*200, Monochrome.

Again, Although it's possible to set the colour of the

pixels, this colour is not saved.

Mode 07h : Hercules 720*348 monochrome.

This is really an odd one out.  Since there is no sure

way to tell if the Hercules card is in graphics mode,

(there IS a technique to see if a TRUE Hercules is in

graphics mode, testing has shown that this technique

does not work for most hercules clones).

SavePCX always assumes graphics mode is active when the

hot key is pressed.  If, at the time the hot key is

pressed, alfanumeric (text) mode is active, the picture

will be saved, but it will probably hold nothing but

garbage.

Mode 0Dh : EGA 320*200, 16 colour.

Mode 0Eh : EGA 640*200, 16 colour.

Mode 0Fh : EGA 640*350, Monochrome.

Mode 10h : EGA 640*350, 16 colour.

Mode 11h : VGA 640*480, Monochrome.

Mode 12h : VGA 640*480, 16 colour.

In all modes from 0Dh through 12H, there's a difference

when you have an EGA or VGA card installed.

If you have an EGA card, the picture will be saved with

standard palette, Page 0.  This is due to the fact that

it's not possible to read out the EGA registers.

If you have a VGA card, SavePCX will correctly save the

active page, correct palette colours and in the case the

screen dimensions have been changed, save the entire

video buffer (It's possible to change the dimensions of

the video buffer but still keeping the same screen size).

Mode 13h : VGA 320*200 256 colour.

Tweaked  : Also called Mode-X, 13h/4, VGAX, Unchained VGA...

This is a very nifty (undocumented) VGA mode, it's a

mode that allows 256 colours but offers you multiple

screen pages (unlike mode 13h).  This mode is used by

some games (Ultima Underworld (Origin), Red Baron

(Dynamix)...) and lots of demos (Cronologia (Cascada),

Vectordemo (UltraForce), Unreal (Future Crew)...).  It's

a hell to program in this mode but the gains can be

enormous.  You can fill the screen up to 4 times as fast

as compared to mode 13h.

SavePCX is probably the ONLY screen capture program

around which saves screens in this mode. Problem is most

games/demo's tend to take complete control of the

keyboard and thus render SavePCX useless in that case.

Nonetheless it's here.

Warning.

--------

To avoid disk drive errors, activation of SavePCX during disk

operations is not recommended (because SavePCX uses the disk

also).  Under normal circumstances, pressing the Hot Key while

you are reading/writing files, shouldn't cause any problems,

SavePCXC will wait until DOS has finished reading/writing. If you

are using a program which reads/writes the disk directly (Like

Disk Optimizers (PCTools Compress, Norton Speeddisk,...)), it's

best to reboot first, to avoid wrecking your harddisk by pressing

the Hot Key.

To avoid a problems with TSR-utilities (not only SavePCX).

1� : Do not load any TSR programs when in a DOS Shell. This will

result in memory fragmentation ("memory allocation error")

when quitting the program you invoked the DOS shell from.

If no error occurs when quitting the program you invoked the

shell from, you will loose a considerable amount of memory

because the program is terminated but is not removed from

memory.

2� : Most programs which have a shell built-in, release all

unused memory.  When trying to return to the program you

shelled from, the computer will hang because the program can

not re-allocate it's freed memory.

3� : Try not to run a program which uses the same combination of

keys as the TSR does.

About the PCX File format.

--------------------------

This paragraph is only for those who are looking to incorporate

pictures in their own programs or want to learn about the structure

of PCX files. If you aren't one of these, just skip this section.

This section requires you to know how to program the video

hardware, because only the PCX format is discussed here.

First of all I'd like to mention that there are a lot of

descriptions of the PCX format around, but none of them (even the

one by ZSoft) is correct on all counts.  The format description

here will probably be the easiest guide around and is based on

a lot of tests done on PCX files created by various programs.

The PCX-pictures created by SavePCX conform to the ZSoft

specification.  Some of the special cases which do not conform to

the ZSoft specification are marked with an asterix (*), it's up to

you to decide to incorporate the exception in your loader.  If you

need a PCX write routine, stick to the PCX convention for maximum

compatibility.

the PCX format supports several types of images, monochrome, 16

colour, 256 colour (8 bit) and even true 24 Bit colour.  Altough

these are the standard (PC) formats, you may have to accomodate for

even more types when you want to write a general purpose PCX

loading/writing routine, images from the amiga have a few more

possibilities (8,32 and 64 colour, 16 colour Bitpacked...).

Most probably however you'll need a PCX routine which

correctly reads one specific type of file.

The PCX file begins with a 128 byte header.  Usually you can ignore

the header, since your images will probably all have the same

resolution and number of colours.  If you want to process different

resolutions or colours, you will need to interpret the header

correctly.  The remainder of the file consists of encoded graphic

data.  The encoding method is a simple byte oriented run-length

technique.

The header (All Bytes are unsigned, Words are signed) :

Offset  Item          Type      Description comment.

------  ------------  --------  -----------------------------------

0       Manufacturer  Byte      Should be 10 (0A Hex).

This is a flag that can be checked

to see if it's a ZSoft compatible

PCX.

(*) even those not conform to the

ZSoft specification have a 10 here,

I haven't found any programs which

have their own value here.

1       Version       Byte      The version number of the PCX file.

0 = V2.5 of PC Paintbrush

2 = V2.8 with palette

3 = V2.8 without palette

4 = PC Paintbrush for Windows (PC

Paintbrush Plus for Windows

uses Ver 5)

5 = Version 3.0 and higher of PC

Paintbrush and PC Paintbrush +

includes Publisher's Paintbrush.

2       Encoding      Byte      1 = .PCX run length encoding

(*) 0 = No crunching. there seem to

be some files around which are not

crunched with the RLE, the graphic

data is stored as a raw image.

3       BitsPerPixel  Byte      Number of bits to represent a pixel

(per plane) for PC 16 colour images

this is 1.

Allowed values are 1, 2, 4, 8.

(*) the 24 bit images have 24 this

is an extention to the PCX format

which has (to my knowing) not yet

been approved of by ZSoft.

(*) other values are possible, some

Amiga PCX files actually have 5 (32

colour) altough most 32 colour

images have 1 but 5 planes.

4       XMin          Word      \  Image dimensions.

6       YMin          Word       \ XMin and YMin are usually 0

8       XMax          Word       / but in the PCC (PCX Blocks) the

10      YMax          Word      /  value is actualy the screen

coordinates from which the block

was taken.

Since counting normally starts

at 0, XMax and YMax will have

one less than the number of

pixels per colum/row.

12      HDPI          Word      Horizontal resolution of image in

DPI

14      VDPI          Word      Vertical resolution of image in DPI

16      Palette       48 bytes  Colour palette setting (See later)

64      Reserved      1 byte    Should be 0.

(*) In Paintbrush this byte usually

holds the number of the video mode

the picture was made in.

65      Planes        Byte      Number of colour planes.

66      BytesPerLine  Word      Number of bytes to allocate for a

scanline plane.  MUST be an EVEN

number.

(*) A *LOT* of programs actually

have an ODD number for bytesPerLine.

oddly enough most PCX loaders don't

load pictures correctly with an odd

number. It's easy enough to

accomodate this in a PCX loader.

There's even a program around

(forgot the name) which can write

a PCX file with ODD BytesPerLine

and can not read the file it just

saved.

68      PaletteInfo   Word      How to interpret palette

1 = Color/BW

2 = Grayscale (ignored in PB IV/IV+)

70      HScreenSize   Word      Horizontal screen size in pixels.

72      VScreenSize   Word      Vertical screen size in pixels.

74      Filler        54 Bytes  Blank to fill out the 128 byte

header.  Should all be 0.

(*) Some programs store proprietary

information here.

When making a PCX loader you can simply ignore most values here.

The ones you need are : BitsPerPixel, XMin, XMax, YMin, YMax,

Planes and BytesPerLine.  Depending on the type of file you'll need

Version and Palette.

Reading .PCX files :

First find the pixel deimensions of the image by calculating

XSize = XMax - XMin +1   and  YSize = YMax - YMin + 1.

Then calculate how many bytes are required to hold on complete

uncompressed scanline :  ScanlineSize = Planes * BytesPerLine.

Because there are always an even number of BytesPerLine, there will

probably be unused data at the end of each plane line.

You can now begin decoding the first scan line, read the first byte

of data from the file.  if the top two bits are set, the remaining

six bits in the byte show how many times to duplicate the next byte

in the file.  If the top two bits are not set, the first byte is

the data itself, with a count of one.

Following procedure can be used :

while not endofscanline do

begin

get a byte (X) from file

if the top two bits of X are 1 then

begin

Count = 6 lowest bits of X

get a new byte (Data) from file

endif

else begin

Count = 1

Data  = X

endelse

Store Count bytes of Data in Output Buffer (or to screen)

endwhile

Continue decoding the rest of the line.  Keep a running subtotal of

how many bytes are moved and duplicated into the output buffer.

When the subtotal equals ScanlineSize, the scanline is complete.

If the subtotal is larger than ScanLineSize, the PCX file is

probably corrupt.

For a picture with more than one plane (16 Colour PC pictures have

4 planes), the planes are stored one after the other. Plane 0

first, Plane 3 last (the sequence of bytes will then be Red plane,

Green plane, Blue plane and Intensity plane).

The RLE can cross the planes, that is, it's not assured there will

be a decoding break at the end of each break (subtotal equals

BytesPerLine).  (*) Most documentations are wrong here, stating a

decoding break MUST occur at the end of each plane.  They thus

write PCX files with a decoding break at the end of each plane,

strangely enough they do manage te READ PCX files without decoding

breaks.  ! SavePCX does not include a decoding break a the end of

each plane.

Use XSize to know what are valid pixel bits (bytes), and

BytesPerLine to shows where each plane ends within the scanline.

Continue decoding the remainder of the IMAGE data, (YSize

scanlines).  Do not just read to end-of-file.  There may be

additional data after the end of the image.

EGA/VGA 16 Colour Palette Information.

In standard RGB format, the data is stored as 16 triplets which can

be found in the PCX header (Palette).  Each triplet is a 3 byte

quantity of Red, Green, and Blue values.  The values can range from

0-255, so some interpretation may be necessary.  On EGA for

example, there are 4 possible levels of RGB for each color.  The

simplest solution is to divide each value by 64 (shift right 6

bits).

On the VGA there are 64 possible levels of RGB for each colour.

You will have to set the VGA DAC palette from color offsets

obtained from the EGA palette registers (get it ? :-) ).

VGA 256 Colour Palette.

The 256 Colour palette is formatted and treated the same as the 16

colour palette, except that it is substantially longer (256

triplets).  The palette (number of colours * 3 bytes per colour) is

appended to the end of the .PCX file, and is preceded by a 12

(0CHex).  Since the VGA DAC expects a palette value to be 0-63 in

stead of 0-255, you need to divide the values read in the palette

by 4 (Shift right 2 bits).

To access the 256 colour  palette.  First check the version number

in the header; if it contains a 5 there is a palette.  Second, read

to the end of the file and count back 769 (3*256+1) bytes. the byte

you should find there is 12, showing the presence of a 256 colour

palette.

These two are the only types of picture which are well defined :

Monochrome images,

4 colour CGA pictures : No matter what's in the Palette, the

picture always seems OK, the palette information is filled with

all kind of different values depending on what program created the

PCX file.  Simply put, treat monochrome images as Black/White

whatever the value in the palette. and treat 4 colour images with

the default CGA colour set (Black, Cyan, Magenta, White).

(*) 32 and 64 colour images have a palette identical to the 256

colour palette, but with 32 or 64 entries in stead of 256.

(*) 8 colour images can have either an 16 colour or 256 colour

compatible palette.  The best way to treat these images is as

follows, if it has a palette appended (like the 256 colour

pictures) use that one, overriding the values in the header. If

there's no appended palette, use the Palette in the Header.

(*) 24 Bit pictures have no palette, the pixels should be treated

as 24-bit (true colour).  To display these images on a standard

VGA, you'll either have to dither the picture or set up a default

palette and calculate an 8 bit value for each 24 bit value (colour

reduction).

To make a PCX Write routine, just reverse it all :-) (come on do

something yourself).  Just incorporate a Version 5 PCX write only,

most PCX loaders read those anyway.

The values in HDPI, VPDI are used for scanned images, depicting the

resolution the picture was scanned at.

HDPI, VDPI, PaletteInfo, VScreenSize, and HScreenSize, just leave

them at 0.  Fill up the Header Palette with a default EGA setting.

Contacting PSI.

---------------

You can contact PSI on following address :

Olivier Reubens

Lippenslaan 205

8300 Knokke-Heist

Belgium

Or you can call the PSI BBS.

FUN-derbird

+32-(0)50-620112

Speeds From 1200 up to 16.8k (MNP4-5, V42 & V42bis)

2.2 Gigabyte online

Lots of Pascal, C, Asm sources

Loads of GIF and MOD

and more.......

Disclaimer

----------

The software, diskettes, and documentation are provided "AS IS".

There is no warranty of merchantability, no warranty of fitness for

a particular use, and no warranty of any kind, either expressed or

implied regarding the software, diskettes, or documentation.

Accordingly, the entire risk as to the use, results and performance

of the software and documentation is assumed by you.  In no event

will Prometheus Systems Incorporated be liable to you or any one

else for any consequential, special, or indirect damages resulting

from possession, use, or malfunction of this product, including

without limitation, damage to property and, to the extent permitted

by law, damage for personal injury, even if Prometheus Systems

Incorporated has been advised of such damages or loss. Some

Countries (states) do no allow limitations on the duration of

implied warranties and/or the exclusion or limitation of incidental

or consequential damages, and thus the preceding limitation and/or

exclusions and limitations may not apply to you.  This warranty

gives you specific legal rights, and you may have other rights

depending on the laws in your country (state).  You agree that the

liability of Prometheus Systems Incorporated arising out of any

kind of legal claim (whether in contract, tort, or otherwise) will

not exceed the amount you originally paid for the use of this

product.