HOW I ADDED A 1.44MB 3.5 INCH FLOPPY TO MY SYSTEM

FIRST WHAT I HAD:

SYSTEM: IBM AT WITH 339 BIOS DATED 11/15/85

OLDER BIOS'S WON'T WORK WITHOUT DEVICE DRIVER -

339 BIOS HAS 1.44MB SUPPORT.

FLOPPY: TOSHIBA FDD 4603A0K - OTHER DRIVES MAY NOT HAVE

AUTO MEDIA SENSING FOR 1.44/720 MEDIA.

ON THE ADAPTER CARD FOR THE RIBBON CABLE

THERE IS A JUMPER ON PIN 34. SET IT TO

THE "B" POSITION TO ENABLE CHANGE-LINE

SUPPORT.

ON THE BOTTOM OF THE DRIVE THERE ARE TWO

JUMPERS. SET ONE IN THE "B" POSITION AND

ONE IN THE "2" POSITION AS SHOWN BELOW

A B  DS

. ._. .

.C. ._. 1

D   2

THE PROBLEM:

WHILE THE AT BIOS HAD SUPPORT FOR THE 1.44MB

DRIVE, MY SETUP DIDN'T. SO, I WROTE A SMALL

PROGRAM CALLED SETUPD TO SET UP JUST THE

FLOPPY DRIVES. A 1.44MB DRIVE IS A TYPE 4.

SETUPD IS INCLUDED HERE WITH MASM SOURCE.

USING THE DRIVE:

I PLUGGED MINE IN AS DRIVE A: AND USED IBM PC/DOS 3.30.

NO DEVICE DRIVERS WERE REQUIRED. TO FORMAT A 1.44MB

FLOPPY, READ, WRITE 1.44MB OR 720K DO NOTHING SPECIAL.

TO FORMAT A 720KB FLOPPY USE THE FOLLOWING FORMAT

COMMAND:

FORMAT A: /N:9 /T:80

DID I DO EVERYTHING RIGHT ?

I DON'T KNOW, BUT IT'S PRETTY HARD TO ARGUE WITH

SUCCESS. I HAVE HAD NO PROBLEMS AT ALL.

WILL MS DOS X.X,PC/DOS X.X, ETC. WORK?

I DON'T KNOW, I DON'T HAVE THEM TO TEST.