----=====================================================----

----====< This file contains additional programming >====----

----====< information for the Pro Audio Spectrum.   >====----

----=====================================================----

Programming PCM stuff... (For the Original Pro Audio Spectrum)

A few considerations when programming the 8253 timer chip.

1. When programming timer 0 (sample rate timer), and

timer 1 (sample buffer count), remember to first

program the shadowed timer control register (138Bh)

to set up the timer mode. This was not well documented

in the early versions of the developers kit manual.

2. A speed related problem has been reported when

programming the 8253 timers. A period of time must

elapse between programming the shadowed timer control

register, and the actual timers. This should be

minimally 2 micro seconds. The best approach is to

perform an OUT instruction, then a couple IN

instructions. The I/O instructions will slow all

processors down to the 8 mhz rate; thereby, forcing

proper timing on all AT class machines.

3. A special note about the sample buffer counter. Normal

PCM record and playback uses mode 2 of the 8253 timer.

This mode only accepts values from 2 through ffff+1 (or

zero for 64k). A value of 1 will not generate the

expected end of buffer interrupt.

4. The description of the sample rate and sample buffer

mask bit in the developers manual, is not clear. These

bits are the enable gates that allow the timers to run.

These bits are set to enable the timers, not to disable

them.

New programming considerations for support the Pro Audio

Spectrum products based upon the MV101 ASIC.

Changes to the 8253 timer chip.

The MV101 ASIC emulates the 8253 timer chip, with

a couple changes. Timer 0 is only an 8 bit value.

When programming the timer for 16 bits, the high

order byte will be ignored.

Timer 1 functionality has been modified. The timer

works the same for 8 and 16 bit PCM on 8 bit DMA

channels, but changes slightly for the 16 bit DMA

channels. The count needs to be divided by 2 when

programming the timer for use on a 16 bit channel.

Single Shot DMA PCM transfers.

When performing DMA transfers, and NOT using the

auto-initialize DMA mode, one special step should

be performed at IRQ time. Upon receiving a Sample

Buffer Count interrupt (buffer complete IRQ), turn

off the PCMEnable bit in the cross channel register.

Once the DMA is programmned for the next block,

and timer 1 reprogrammed for the new block length,

the PCMEnable bit can be turned back on.

DMA & IRQ channel support.

The Pro Audio spectrum product line, based upon the

MV101 ASIC, supports the high order DMA and IRQs

of the AT class machines. Please add support for

these channels. Example code can be found in

MVSOUND.ASM in \PAS\SUBS\MVSOUND.

Board I/O relocation.

The MV1010 ASIC allows the Pro Audio to be relocated

to alternate base I/O addresses. For example, the

card can be moved, from 388 to 384. This affects all

native Pro Audio addresses, such as the filter, pcm,

interrupt, fm, and MIDI addresses. Use the function,

"mvGetHWVersion" to determine the current I/O address.

For coding examples, checkout MVSOUND.ASM in the

subdirectory, \PAS\SUBS\MVSOUND. This code calls

mvGetHWVersion, then uses a special memory variable,

"_MVTranslateCode" to adjust the original address.

Stereo 2OP FM Synthesis.

The new OPL3 4OP FM chip does not exactly emulate

two 2OP FM chips. Two programs, PAN2OP.C and PHASE.C,

located in \PAS\SUBS\FM, show how to perform stereo

2OP on either hardware platforms.